Disposal of radioactive material

Management licence holder’s obligations Radiation Act 2005

OFFICIAL

# Introduction

The Victorian Radiation Act 2005 (the Act) has the objective of protecting the health and safety of persons and the environment from the harmful effects of radiation. The Department of Health (Department) administers this legislation.

The Act seeks to fulfil this objective by establishing a licensing framework to regulate the conduct of radiation practices and the use of radiation sources.

Any person who conducts a radiation practice must hold a management licence (unless exempted from that requirement). The management licence holder must comply with every condition of their licence.

# Scope

All management licences issued by the Department include a condition which requires a management licence holder whose licence permits disposal of radioactive material to meet the requirements described in this document.

Disposal in this context includes:

relocation of a radiation source to a destination outside Victoria; and

relinquishing possession of a radiation source through sale of the radiation source or giving away of the radiation source or any other means.

These restrictions do not apply to the disposal of bodily wastes to sewer by a person who, as part of a diagnostic or therapeutic medical procedure, has previously had an administration or implantation of radioactive material.

# What methods are available for the disposal of radioactive material?

The available methods of disposal are:

For short-lived unsealed radionuclides, the ‘delay and decay’ method;

For long-lived sealed sources, disposal via export;

For short-lived unsealed radionuclides, disposal to sewer;

For short-lived radionuclides, disposal by release to air;

For any radioactive material, via transfer of ownership;

For any radioactive material, via relocation outside Victoria.

# What is the ‘delay and decay’ method of disposal?

- Put simply, this is the careful storage of the radioactive waste until such time as it no longer meets the definition of ‘radioactive material’ under the Act. This in practice only works for short-lived radioisotopes and any material that may have contact with the radioactive material e.g. syringes, dressings etc.

- There are a number of steps that you must take to ensure that you are holding the material for sufficient time for the decay to have resulted in the material no longer meeting the definition of radioactive material. These include:

Ensuring that isotopes cannot be mixed. One box = One isotope

- You need to calculate that date when the waste will be below the prescribed activity or activity concentration that defines radioactive material under the Radiation Action Act 2005. As a guide, for most short-lived radionuclides in small quantities, you will need to retain the material for a time equal to at least 10 half-lives.

Clearly label your waste containers

Do not mix solid waste and liquid waste together

- Different guidelines apply based on the physical property of the waste.

- Once you are satisfied that the waste material is no longer radioactive under the Act’s definition of radioactive material, you can commence the disposal process. The following advice is provided to assist you in this process:

Remove radioactive and hazardous material markings and labels on regular waste

- All waste which can be disposed as regular waste because it no longer meets the definition of radioactive material under Victorian law may be sent to landfill. You must ensure that radioactive markings from tape, labels, vials and other material have been removed before placing materials inside a container. You will also need to ensure that you comply with the Environment Protection Act 1970 in regard to whether the other characteristics of the waste meet the Environment Protection Authority’s (EPA) definition of prescribed waste. Contact the EPA on 9695 2722 to discuss the Victorian requirements for prescribed waste.

Consider removing lead pigs & isotope protective casings

- The presence of the pigs could interfere with the tests of activity of the waste material.

# Disposal of sealed sources via export

#### Disposal via a Victorian Serviced Provider

- The Department has licensed a small number of Victorian based companies to dispose of sealed sources and

- of sealed source apparatus by export to facilities located in other countries. In some cases, the sealed

- sources once exported are actually reprocessed or recycled by the international facility so that they can be

- used again by another party.

##### Responsibilities of a Management Licence holder authorised to possess a sealed source

- If you wish to dispose of a sealed source or of a sealed source apparatus by export by using a ‘service

- provider’, then you must:

Contact one of the companies (‘service providers’) who are licensed in Victoria to provide this service. A list of service providers authorised to facilitate the disposal of radioactive material licensed in Victoria is available at: https://www.health.vic.gov.au/publications/service-providers-authorised-to-facilitate-the-disposal-of-radioactive-material

- Please telephone 1300 767 469 or email Radiation.Safety@health.vic.gov.aushould you require further details regarding service providers in Victoria

Apply to the Department for the approval to dispose of a sealed source or a sealed source apparatus using the Disposal application form – Management Licence holder who is authorised as a source disposal facilitator.

Dispose of the sealed source only when you have received approval in writing from the Department for the disposal of the sealed source.

If the disposal involves taking possession of the sealed source, then take possession of the sealed source only when you have received approval in writing from the Department for the disposal of the sealed source

- Prior to the sealed source being delivered to you, you may also need to vary your licence to authorise the possession of the sealed source.

- In the event that the export process has not occurred within three months of your taking possession of the sealed source you must notify the management licence holder who sought your services to dispose of the source. You must provide reasonable assistance to ensure that the management licence holder takes possession of the sealed source as soon as practicable.

- If the disposal involves taking possession of the sealed source, then you must notify the Department regarding the possession in accordance with a general condition on your management licence. When you have disposed of the sealed source, you must notify the Department of the disposal in accordance with the requirements specified under the section titled ‘Notification of the disposal’ on page five of this document.

#### Disposal via the Management Licence holder authorised to possess a sealed source

##### Responsibilities of a Management Licence holder authorised to possess a sealed source

- If you hold a management licence authorising the possession of a sealed source which you wish to dispose of by export or via an interstate Service Provider, then you must:

Apply to the Department for the approval to dispose of a sealed source or a sealed source apparatus using the Disposal application form - Management Licence holder who is authorised to possess a sealed source or a sealed source apparatus.

Dispose of the sealed source only when you have received approval in writing from the Department for the disposal of the sealed source.

- In the event that the export process has not occurred within three months of the sealed source being transported to the service provider’s premises the service provider is required to surrender the sealed source to you. You must take possession of the sealed source as soon as practicable after being advised by the service provider that the disposal cannot occur.

- The management licence holder must ensure that the Department is notified of the disposal in accordance with the requirements specified under the section titled ‘Notification of the disposal’ on page six of this document.

##### Responsibilities of a Management Licence holder authorised as a disposal facilitator

- If you are a service provider that is authorised to facilitate the disposal of a sealed source or a sealed source

- apparatus by export, then prior to facilitating the disposal of a sealed source or a sealed source apparatus

- you must:

Apply to the Department for the approval to dispose of the sealed source using the Disposal application form - Management Licence holder who is authorised as a source disposal facilitator.

Dispose of the sealed source only when you have received approval in writing from the Department for the disposal of the sealed source,

If the disposal involves taking possession of the sealed source, then take possession of the sealed source only when you have received approval in writing from the Department for the disposal of the sealed source.

- Prior to the sealed source being delivered to you, you may also need to vary your licence to authorise the possession of the sealed source.

- In the event that the export process has not occurred within three months of your taking possession of the sealed source you must notify the management licence holder who sought your services to dispose of the source. You must provide reasonable assistance to ensure that the management licence holder takes possession of the sealed source as soon as practicable.

- If the disposal involves taking possession of the sealed source, then you must notify the Department regarding the possession in accordance with a general condition on your management licence. When you have disposed of the sealed source, you must notify the Department of the disposal in accordance with the requirements specified under the section titled ‘Notification of the disposal’ on page six of this document.

#### Disposal by the Management Licence holder authorised to possess a sealed source

##### Responsibilities of a Management Licence holder authorised to possess a sealed source

- If you hold a management licence authorising the possession of a sealed source which you wish to dispose of by export or via an interstate Service Provider, then you must:

Apply to the Department for the approval to dispose of a sealed source or a sealed source apparatus using the Disposal application form - Management Licence holder who is authorised to possess a sealed source or a sealed source apparatus.

Dispose of the sealed source only when you have received approval in writing from the Department for the disposal of the sealed source.

- The management licence holder must ensure that the Department is notified of the disposal in accordance

- with the requirements specified under the section titled ‘Notification of the disposal’ on page six of this

- document.

# Disposal of radioactive material via transfer of ownership

- Radioactive material may be disposed of by transferring ownership of the radioactive material. This transfer

- may occur either by selling the material or by giving the material away subject to the following requirements:

Apply to the Department for the approval to dispose of a sealed source or a sealed source apparatus using the Disposal application form - Management Licence holder who is transferring ownership of a sealed source or sealed source apparatus.

Dispose of the sealed source only when you have received approval in writing from the Department for the disposal of the sealed source.

- The management licence holder must ensure that the Department is notified of the disposal in accordance

- with the requirements specified under the section titled ‘Notification of the disposal’ on page six of this

- document.

# Disposal by relocation outside Victoria

- Radioactive material may also be disposed of by relocation of the material outside Victoria without changing

- ownership.

- For example, a sealed source apparatus may be moved out of Victoria, either temporarily or permanently, for

- use by the licence holder outside Victoria.

- In the case of high consequence sealed sources (Category 1, 2 or 3 sealed sources) as defined in the

- Radiation Act 2005 you must:

Apply to the Department for the approval to dispose of a sealed source or a sealed source apparatus using the Disposal application form - Management Licence holder who is relocating a Security Category 1, 2 or 3 sealed source or a Security Category 1, 2 or 3 sealed source apparatus interstate.

Dispose of the sealed source only when you have received approval in writing from the Department for the disposal of the sealed source.

- Disposal of a source which is not a high consequence sealed source, by relocation outside Victoria, does not require prior approval. The management licence holder must ensure that the Department is notified of the disposal in accordance with the requirements specified under the section titled ‘Notification of the disposal’ on page six of this document.

- Note also that in the case of temporarily relocated radiation sources that the licence holder must also notify the Department on regaining possession of the source in Victoria.

# Disposal of unsealed radionuclides, such as radiopharmaceuticals, to sewer

- Disposal of unsealed radionuclides to sewer must not occur unless:

In any 24-hour period, the average concentration at the point of discharge from the premises into the sewerage system does not exceed the concentrations listed in Column 2 of Table 1 when diluted by the average daily concentration of sewage discharged from the premises;

Records are kept of the nature and amounts of radioactive material discharged from the premises;

The disposal has been specifically approved in writing by the Department.

- Approvals may also be required by the relevant water authority for the discharge of material into the sewerage

- system.

# Disposal to air

- Disposal of radioactive material to air must not occur unless the average concentration in any 24 hour period

- at the point of discharge from the premises does not exceed the limits specified in Column 3 of Table 1 below.

- No radioactive material that exceeds the limits specified in Column 3 of Table 1 below can be disposed of by

- air unless you have notified the Department in writing of the details of the proposed disposal and the

- Department has approved the proposed disposal in writing.

# Disposal to landfill

No radioactive material may be disposed of to landfill unless you have notified the Department in writing of the details of the proposed disposal and the Department has approved the proposed disposal in writing. At the time of writing there is no available option to dispose of radioactive material to landfill

# Notification of the disposal

The management licence holder must ensure the Department is notified of the disposal using the on-line Notification Form within:

- 24 hours for the disposal of high consequence sealed sources as defined by the Radiation Act 2005; and

14 days for other types of sealed sources or sealed source apparatus or other types of radioactive material.

Note that no notification is required to the Department of the disposal of unsealed radionuclides or radiopharmaceuticals due to the short-lived nature of these forms of radioactive material.

# Further information

Visit the Disposal of radioactive material and X-ray equipment page on the Radiation website.

Email the Radiation Team

# Discharge limits

Table 1 Discharge limits

| Column 1 | Column 2 | Column 3 |
| --- | --- | --- |
| Radionuclide | Limits on Discharge to Sewerage system (Bq.m-3) | Limits on Discharge to Air (Bq.m-3) |
| 3H | 3 x 107 | - 4 x 102 |
| 14C | 2 x 106 | - 2 x 101 |
| 18F | 3 × 107 | - 2 x 103 |
| 22Na | 4 × 105 | - 9 x 101 |
| 24Na | 3 × 106 | - 4 x 102 |
| 32P | 6 × 105 | - 3 x 101 |
| 33P | 6 × 106 | - 8 x 101 |
| 35S | 2 × 106 | - 6 x 101 |
| 36Cl | 2 × 106 | - 2 x 101 |
| 45Ca | 2 × 106 | - 3 x 101 |
| 51Cr | 4 × 107 | - 3 x 103 |
| 59Fe | 8 × 105 | - 3 x 101 |
| 57Co | 7 × 106 | - 1 x 102 |
| 60Co | 7 × 105 | - 3 x 100 |
| 63Ni | 9 × 106 | - 9 × 101 |
| 65Zn | 4 x 105 | - 6 x 101 |
| 67Ga | 7 x 106 | - 5 x 102 |
| 85Kr | N/A | - 1 x 105 |
| 89Sr | 5 x 105 | - 1 x 102 |
| 90Sr | 5 x 104 | - 5 x 100 |
| 90Y | 5 x 105 | - 8 x 101 |
| 99Mo | 2 x 106 | - 1 x 102 |
| 99Tc | 2 x 106 | - 3 x 101 |
| 99mTc | 6 x 107 | - 6 x 103 |
| 125I | 9 x 104 | - 8 x 101 |
| 131I | 6 x 104 | - 5 x 101 |
| 137Cs | 1 x 105 | - 3 x 101 |
| 147Pm | 5 x 106 | - 2 x 101 |
| 153Sm | 2 x 106 | - 2 x 102 |
| 192Ir | 1 x 106 | - 2 x 101 |
| 197Hg | 6 x 106 | - 4 x 101 |
| 203Hg | 7 x 105 | - 5 x 101 |
| 201Ti | 1 x 107 | - 3 x 103 |
| 204Ti | 1 x 106 | - 3 x 102 |
| 241Am | 7 x 103 | - 3 x 10-3 |
| 252Cf | 2 x 104 | - 6 x 10-3 |
| Alpha-emitting radionuclides not listed in this Table | 1 x 104 | - 1 x 10-2 |
| Radionuclides that are not alpha-emitting and not listed in this Table | 1 x 105 | - 1 x 101 |
| Radionuclides of natural origin | 1 x 103 | - 1 x 10-2 |

To receive this document in another format, email Radiation Team <Radiation.Safety@health.vic.gov.au>. Authorised and published by the Victorian Government, 1 Treasury Place, Melbourne. © State of Victoria, Australia, Department of Health, January 2022. Available at Disposal of radioactive material and X-ray equipment page on the Radiation website. <https://www.health.vic.gov.au/radiation/disposal-of-radioactive-material-and-x-ray-equipment>

Where the material contains a mixture of radionuclides, the limit is determined from the sum of the weighted activities or concentrations respectively for each radionuclide in the mixture, which must not exceed 1.