Guidance - Water Quality Annual Report

Section 26 of the Safe Drinking Water Act 2003 Regulations 21 & 22 of the Safe Drinking Water Regulations 2025

Contents

Preface	4

Introduction	5

What to include in the annual report?	5

Glossary of terms	6

How to use this guidance?	6

Lodgement of the annual report	6

Further information	6

Part 1: Water supplier content guide	7

Overview	7

Water supply systems	7

Water sampling areas: Regulation 21(a)	7

Source water	7

Drinking water treatment processes: Regulation 21(f)	8

Issues	9

Emergency, incident and event management Regulation 21 (b) & Regulation 21 (c)	9

Issues	10

Presentation of information	10

Drinking water quality standards Regulation 21(g), (h), (i) (j)	11

Presentation of information	13

Drinking water quality standard not met & actions undertaken	14

Presentation of Regulation 16 Drinking water quality standards	15

Aesthetic characteristics: r. 21 (n)	16

Presentation of other water quality standards (microbial, chemical or physical characteristic) - r. 16(2)(b) and (c)	17

Water quality complaints r. 21(m)	18

Risk management plan audit results r. 21(e)	21

Undertakings r. 21(d)	21

Aesthetic standard variations r. 21(k)	22

Exemptions r. 21(l)	22

Regulated water r. 21(o)	22

Part 2: Water storage manager content guide	24

Overview	24

Water supply systems	24

Source water	24

Drinking water treatment processes r. 22(e)	25

Summary of any issues	26

Emergency incident and event management r.22(a) & 22(b)	26

Known or suspected contamination reported under s. 22	26

Issues from emergency or incident	27

Presentation of information	27

Risk management plan audit results r. 22(d)	27

Undertakings r. 22(c)	28

Exemptions r. 22(f)	28

Appendix 1: Example template for reporting drinking water treatment processes	29

Appendix 2: Checklist for water suppliers	30

Trihalomethanes	31

pH	31

Detail variations to the sampling frequency specified in the drinking water sampling program	31

Turbidity	31

Other water quality standards r. 16 (microbial, chemical, or physical characteristics)	32

Drinking water quality standards not met & actions undertaken	32

Aesthetic characteristics r. 16(1)(b)	32

Water quality complaints r. 21(m)	32

Aesthetic characteristics r. 21(n)	32

Risk management plan audit results r. 21(e)	33

Undertakings r. 21(d)	33

Aesthetic standard variations r. 16(1)(c) r. 21(k)	33

Exemptions r. 21(l)	33

Regulated water r. 21(o)	33

Appendix 3: Checklist for water storage managers	34

Overview	34

Drinking water treatment processes r. 22(e)	34

Emergency incident and event management r. 22(a) & 22(b)	34

Risk management plan audit results r. 22(d)	34

Undertakings r. 22(c)	35

Exemptions r. 22(f)	35

To receive this publication in an accessible format phone 1300 761 874, using the National Relay Service 13 36 77 if required, or email water@health.vic.gov.au

Authorised and published by the Victorian Government, 1 Treasury Place, Melbourne

© State of Victoria, Department of Health, July 2026

# Preface

Victoria‘s Safe Drinking Water Act 2003 (the Act) and Safe Drinking Water Regulations 2025 (the Regulations) provide a catchment-to-tap risk management framework to ensure the supply of safe, high quality drinking water across the state. The Department of Health (the Department) administers this legislation.

In summary, the legislation requires water suppliers and water storage managers (collectively referred to as water agencies) to:

- Develop and implement risk management plans

- Meet water quality standards

- Disclose information to the department and the public.

Disclosing information to the public ensures accountability and safeguards the integrity and trust in the state-owned water agencies, assisting to keep community confidence in Victoria‘s drinking water supplies.

One form of public disclosure is by reporting; section 26 of the Act requires water agencies to provide an annual report on the quality of drinking water and regulated water for each financial year. This annual report must be provided to the Department no later than 31 October each year and be made available to the public on or after the day the report is provided to the Department.

The Department has a role under the Act to promote public awareness and understanding of drinking water quality issues; water agencies’ annual reports are an important mechanism for communicating drinking water quality performance and supporting informed community understanding of drinking water quality.

This document provides guidance as to how water suppliers and water storage managers can meet the annual reporting requirements under the Act and the Regulations. This guidance sets out the minimum requirements needed to comply. Water agencies are encouraged to provide further information and showcase achievements relating to drinking water and regulated water, to improve the public‘s awareness and understanding of the activities conducted and to emphasise the important role water agencies play in protecting and promoting public health.

# Introduction

Section 26 of the Act requires water suppliers and water storage managers to provide the Secretary of the Department a report on the issues relating to the quality of drinking water and regulated water each financial year. This report must detail compliance with the requirements outlined in the Act and r. 21 and r. 22 of the Regulations.

In accordance with s. 26(3) of the Act, the Secretary may by written notice published in the Government Gazette require additional information in an annual report.

Safe Drinking Water Act 2003 Section 26 – Water suppliers or water storage managers to provide annual report - A water supplier or water storage manager must give the Secretary a report, in respect of each financial year, on the issues relating to the quality of drinking water and regulated water that are specified by the regulations for the purposes of this section. - A water supplier or water storage manager must include in the report any other details that are required by the regulations or by the Secretary under subsection (3). - The Secretary may, by written notice published in the Government Gazette, require water suppliers or water storage managers to include specified details in any report required by this section. - The water supplier or water storage manager must give the report to the Secretary no later than 31 October. - The water supplier or water storage manager must ensure that copies of the report are made available to the public on or after the day the report is given to the Secretary.

## What to include in the annual report?

The Department gives guidance to water agencies on what information is to be included in drinking water quality annual reports. Reports that follow this guidance are easier to assess for compliance against the safe drinking water legislation.

Annual reports are to cover one financial year, the reporting period being 1 July–30 June. The Regulations came into effect on 6 July 2025, which means the Safe Drinking Water Regulations 2015 were still in effect for the first five days of the 2025-26 Annual Report period. Water suppliers and water storage managers should:

- include data for these five days in their Annual Report and include this data in the assessment of compliance against the 2025 Regulations.

Water agencies should use the annual report as an opportunity to describe their risk management practices, any associated water quality issues which arose over the reporting period and proactive actions taken to protect water quality from catchment-to-tap.

Information on how interested readers may receive further information from the water agency (such as a contact number or web address) should also be included. A statement on how the water agency complies with s. 23 of the Act in relation to public disclosure of water quality monitoring information should also be provided.

Safe Drinking Water Act 2003 Section 23 – Water quality monitoring information to be made publicly available - A water supplier must make available for inspection by the public the results of any water quality monitoring program that it conducts on any drinking water supplied by it. - The results must be made available within 7 days after they are compiled. - A water supplier must not publish any information in purported compliance with this section that it knows is false or misleading in a material detail without including with the information details of the defect in the information

## Glossary of terms

A glossary of terms to give clarity on technical issues and terms (including acronyms) should be included.

## How to use this guidance?

Part 1 gives guidance for water suppliers in accordance with r. 21 of the Regulations. Part 2 gives guidance for water storage managers relating to r. 22. Checklists are provided in the appendices to check all required content has been included in the report.

Tables provided in this guidance highlight the minimum content required. Water agencies may choose to include further information in their annual report.

## Lodgement of the annual report

An approved finalised draft report can be submitted for review to the Department by 15 September, allowing sufficient time for the department’s review and for the water agency to make any required changes.

Section 26(4) of the Act requires the water supplier or water storage manager to provide the report to the Department no later than 31 October.

As required by S. 26(5) of the Act, the final report must be made available to the public on or after the day it is provided to the Department. This requirement may be met by having the report publicly available for download from the water agency‘s website, and hard copies mailed out upon request.

An electronic copy of the final report in PDF format should be emailed to water@health.vic.gov.au.

## Further information

Contact via email (water@health.vic.gov.au). Website:

<https://www.health.vic.gov.au/water/guidance-notes>

# Part 1: Water supplier content guide

## Overview

Describe the overall water supply systems (from catchment-to-tap), the commitment to drinking water quality and the preventive measures applied to manage drinking water quality risks.

## Water supply systems

Provide a summary of each drinking water supply system. This should include a clear map or diagram of the water supply systems serviced. Detailed process control schematics are not required.

Include any major changes to source water supply arrangements during the reporting period from the previous reporting period and any other information the water supplier considers relevant.

A table similar to Table 1 can be provided to assist readers to better understand the water supply systems. Consider including water supply system information such as water sampling areas, population supplied, source water, storage reservoirs and treatment plants.

Table 1: Example table for water supply systems

| Water sampling area | Population supplied | Source water | Storage | Treatment plant |
| --- | --- | --- | --- | --- |
| Area A | 2500 | X river | Y storage | Z treatment plant |

Water sampling areas in this table may be listed in alphabetical order, by common source or by water treatment plant. If possible, the information should be presented as a single table. During the reporting period there may be instances where water sampling areas have been changed. Include information on the date of the changes, either as a footnote to the table for minor changes or in accompanying text where further explanation is needed.

The population supplied for each water sampling area should be rounded to the nearest 10. If converting the number of connections to population served, the calculation used should be explained.

## Water sampling areas: Regulation 21(a)

Regulation 21 (a) requires details of any new water sampling areas and any variations to water sampling areas specified in the water supplier’s risk management plan to be included. Regulation 11 and r. 12 require a water supplier to provide a written notice to the Secretary within 10 days for a new or varied water sampling area. Any updates to the water sampling area must be updated in the water supplier’s risk management plan.

## Source water

The water supplier should describe the various source waters used in its water supply systems. Include the catchment management authorities and water storage managers managing water prior to supply to the water supplier. This may include in general terms the nature, purpose and comprehensiveness of the raw water sampling program (e.g. raw water storages, natural water courses, catchments or reservoirs) for pathogens, harmful algae, chemicals and radiological parameters. Detail the interactions with other agencies on catchment management activities. For water suppliers where drinking water treatment processes were applied to raw water, give details of methods used in sanitary surveys and to quantify microbial hazards.

## Drinking water treatment processes: Regulation 21(f)

Safe Drinking Water Regulations 2025 Regulation 21 – Additional details to be included in annual reports of water suppliers. (f) If the water supplier applied a drinking water treatment process to water supplied by the water supplier during the financial year— - a summary of how the water was disinfected or treated; and - a summary of any other processes applied to the water; and - a summary of any issues that arose as a consequence of applying those processes; and - a list of all chemicals and other substances used to disinfect or treat the water

Regulation 21 (f) requires the inclusion of details of the drinking water treatment processes applied to water supplied by a water supplier in the annual report. Regulation 21 (f)(ii) requires information to be included about any other processes that were applied. Regulation 21 (f) (iii) requires information on any issues that arose as a consequence of applying those processes and Regulation 21 (f) (iv) requires a list of all the chemicals and substances used. A template with examples is provided in Appendix 1.

Information describing the treatment processes, changes made and any periodic treatment activities are to be included for each drinking water supply system. This description should extend from the point raw water enters the treatment plant through to the customer tap (catchment-to-tap).

Information on the treatment processes and the chemicals associated with those processes (including water-soluble packaging that may be used to add chemicals) may be presented as shown in Table 2. Changes in the water treatment process between the previous and this reporting period should be clearly highlighted.

Table 2: Example table for drinking water treatment processes - Water supplier

| Water sampling area | Treatment plant | Treatment process^ | Added substance/s | Comments |
| --- | --- | --- | --- | --- |
| Area A | B treatment plant | Membrane ultrafiltration Chloramination Chlorination Fluoridation | Sodium hypochlorite Aqueous ammonia Fluorosilicic acid | Changed chlorine disinfection method from Chloramination to chlorination from 6 April to 2 May to reduce nitrification. Fluoride dosing installed (December) |

^Explain treatment processes in the annual report‘s glossary if necessary.

## Issues

Regulation 21 (f) (iii) requires any issues from the application of treatment processes to be provided.

These issues may include situations where there was insufficient treatment due to treatment process breakdowns or where there was loss of control of treatment processes leading to the potential or actual exceedances of drinking water quality standards.

Examples include clarifier malfunction leading to sub-optimal coagulation/flocculation, continued operation of treatment processes following breaches of critical control point critical limits, lengthy outages of the fluoride dosing system, alternative drinking water supply sought by the water supplier due to poor raw water quality or delays with treatment process repairs.

Details should include information on the effectiveness of any interim and long-term interventions, timing of remedial works or activities and any impacts on the quality of drinking water supplied.

Issues reported under s. 22 of the Act can be acknowledged in this section and cross referenced to details provided under the Emergency incident and event management section of the report.

If there were no issues, indicate this was the case.

## Emergency, incident and event management Regulation 21 (b) & Regulation 21 (c)

Safe Drinking Water Regulations 2025 Regulation 21 – Additional details to be included in annual reports of water suppliers. - If an emergency or incident occurred during the financial year that affected the quality of water supplied by the water supplier, information about actions taken by the water supplier in response to that emergency or incident; and - any issues that arose as a consequence of the actions referred to in paragraph (b)

Regulation 21 (b) and (c) requires information, actions and issues relating to emergencies, incidents and management of drinking water quality events by a water supplier to be included in its annual report. Examples include E. coli detections, boil water advisories, critical control point breaches, treatment (including disinfection) failures, dirty water caused by burst water mains which resulted in widespread public complaint and cross connection of recycled water to drinking water.

Details reported for any emergency, incident or event, which affected both the water supplier and water storage manager, should not contradict with information contained in each water agency‘s annual report.

Known or suspected contamination reported under Section 22

Information on known or suspected contamination reported under s. 22 must be included.

Safe Drinking Water Act 2003 Section 22 – Officer to report known or suspected contamination - This section applies if an officer of a water supplier, water storage manager or council believes or suspects, on reasonable grounds, that water supplied, or to be supplied, for drinking purposes— - may be the cause of an illness; or - may be the means by which an illness is being, has been or will be, transmitted; or - may contain any pathogen, substance, chemical or blue-green algae toxin, whether alone or in combination, at levels that may pose a risk to human health; or - may cause widespread public complaint. - On forming that belief or suspicion, the officer must immediately report his or her belief or suspicion to the Secretary and must make the report in the form required by the Secretary..

For each report made to the Department under s. 22, provide context of the event in relation to the affected water sampling areas. Clearly describe the event and include the issues identified, the corrective actions taken to reduce risks, and any preventive actions initiated to minimise reoccurrence.

## Issues

Under r. 21 (c), any issues that occurred from any actions referred to in r. 21 (b) must be detailed.

## Presentation of information

Each report should be clearly identified, described in narrative text and contained in the main body of the report. These reports  are not to be included solely in footnotes. A table summarising the information may be included.

The following information should be provided where applicable:

- The date, estimated duration and the nature of the emergency, incident, or event

- Affected drinking water supplies and water sampling areas

- The corrective actions taken to reduce and manage risks

- Preventive measures implemented to minimise the risk of reoccurrence, including those where the water supplier has implemented learnings across other assets within the network

- The nature of any communication with affected communities or the public such as via social media if a drinking water advisory was issued.

## Drinking water quality standards Regulation 21(g), (h), (i) (j)

Regulation 21(g) specifies that a water supplier‘s annual report must contain information relating to compliance with the drinking water quality standards specified in r. 16 for the purposes of s. 17(1) of the Act.

Safe Drinking Water Regulations 2025 Regulation 21 – Additional details to be included in a water supplier’s annual report h) details of the water supplier’s compliance with regulation 17 during the financial year; and i) if water supplied by the water supplier during the financial year, did not comply with a drinking water quality standard, information about actions taken by the water supplier in relation to that water; and j) in relation to the analysis of water samples collected by the water supplier during the financial year – - an evaluation of analysis results and information in respect of those water samples; and - a comparison of those results and information with corresponding results and information in respect of water samples collected during the previous 2 years.

Information relating to drinking water quality standards can be found here: Guidance – drinking water quality standards <https://www.health.vic.gov.au/water/guidance-notes>.

Under r. 21 (h) details of the water supplier’s compliance with Regulation 17 during the financial year must be presented.

Where water quality standards had not been met, r. 21 (i) requires information on the corrective and preventive actions taken by the water supplier on receiving the non-compliant results, to be included in the report.

Regulation 21 (j) requires an evaluation of analysis results and information for collected water samples for the reporting period and a comparison to the corresponding information and results for the previous two financial years to be included in the report. If a larger dataset is available and its inclusion would assist in understanding this information, it may also be included.

Water suppliers should include a discussion or table of any changes made to the water sampling program (Regulation 14) since the previous reporting period, indicating the reasons for any changes made. Water quality analysis results from non-routine water sampling that is not part of the water sampling program, such as incident investigations or projects, should not be included.

Guidance relevant to notification and reporting requirements under sections 18 and 22 of the Safe Drinking Water Act 2003 and regulation 20 is available here: Guidance - notification and reporting requirements  <https://www.health.vic.gov.au/water/guidance-notes>

Safe Drinking Water Regulations 2025 Regulation 16 – Drinking water quality standards (1) For the purposes of section 17(1) of the Act, the specified quality standards for drinking water are - - the health-based drinking water quality standards specified in subregulation (2); and - the aesthetic drinking water quality standard specified in subregulation (3); and - on and from 6 January 2028, the aesthetic drinking water quality standards specified in subregulation (4). For more details refer to Guidance – drinking water quality standards <https://www.health.vic.gov.au/water/guidance-notes>

Safe Drinking Water Regulations 2025 Regulation 17 – Frequency of collecting drinking water samples A water supplier must ensure that samples of drinking water are collected from each location specified in the water supplier's water sampling program - - for a water sample that is tested for a sample characteristic to which a determination under regulation 18 applies, at least as often as the frequency specified in that determination; and - in any other case, at least as often as the frequency specified in the water supplier's water sampling program.

Safe Drinking Water Regulations 2025 Regulation 19 – Analysis of drinking water samples A water supplier must ensure that each sample of drinking water collected from each location specified in the water supplier's water sampling program is analysed by an accredited testing organisation – - for each characteristic that is specified in the water supplier's water sampling program; and - at the testing frequency that is specified in the water supplier's water sampling program. Accredited testing organisation means an organisation that holds accreditation from NATA in relation to testing potable water for characteristics specified in a water supplier's water sampling program.

Safe Drinking Water Regulations 2025 Regulation 20 – Requirement to give analysis results to Secretary (1) Within 10 days after a water supplier receives analysis results, the water supplier must give a written summary of the results to the Secretary if those results indicate that - - a sample characteristic of the water contravenes a drinking water quality standard; or - in the reasonable opinion of the water supplier, the level of a characteristic measured in the water may pose a risk to public health or result in widespread public complaint.

## Presentation of information

Table 3 provides a format in which drinking water quality standard results can be presented. Results should be reported to the same number of decimal places or significant figures, relevant to the reported water quality standard. Rounding is not a valid method to determine compliance with water quality standards or other health guideline values. Rounding advice is provided in Guidance – drinking water quality standards <https://www.health.vic.gov.au/water/guidance-notes>.

Table 3: Drinking water quality standards reporting format

| Water sampling area | Frequency of sampling | Number of samples | Drinking water quality standard (mg/L) | Maximum (mg/L) | Average (mg/L) | Number of samples where standard was not met (s. 18) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Area A | Monthly | 12 | 1.0 | 0.2 | 0.1 | 0 |  |
| Area B | Monthly | 11* | 1.0 | 1.1 | 0.6 | 1# | [Details] |

Standard: Parameter X in drinking water should not exceed 1.0 mg/L.

* One monthly sample was missed from area B due to scheduling error.

# Discuss this event under ‘Drinking water quality standard not met & actions undertaken’ section underneath the standard results table.

Water sampling areas in this table may be listed in alphabetical order, or by common source or water treatment plant. For areas that have been merged or split during the reporting period, report the data for each water sampling area pre- and post-merger or split and include explanatory text.

Any variations to the sampling frequency specified in the drinking water sampling program or gazetted in accordance with r. 18 must be noted. Where the frequency has changed, provide an explanation.

Non-compliance with r. 17 and r. 19, such as missed samples or where samples were collected but not analysed must also be detailed.

Analysis results must include all drinking water samples specified in the drinking water sampling program irrespective of the sample location type (e.g. customer tap, water storage tank, entry points). For water suppliers where sampling was conducted at water storage tanks not located within water sampling areas, results must be reported in the water sampling areas to which the water storage tank supplies.

That is, if a water storage tank supplies to two or more water sampling areas, results must be reported in all the relevant water sampling areas, as water in storage is representative of the water supplied to all relevant areas. Include explanatory text as needed.

Water suppliers must analyse each water quality standard’s results with the previous two financial years. This comparison may be presented numerically or by describing whether water quality performance has improved, declined or remained the same over the reporting periods.

This may be included in the table or presented separately as part of the discussion for each standard. Different methods of analysis may be chosen by the water supplier to assist with presenting the analysis. Detail about any works, programs and projects that highlight continuous improvement should also be included.

If all drinking water supplies met the relevant drinking water quality standard, and there were no reports made under s. 22 relevant to the standard for any of the required samples collected and analysed, indicate this was the case.

## Drinking water quality standard not met & actions undertaken

A Drinking water quality standard not met and actions undertaken section underneath the standard table must be included if any water samples did not meet the water quality standard under s. 18 of the Act during the reporting period.

Safe Drinking Water Act 2003 Section 18 – Notification required if non-complying water supplied A water supplier must notify the Secretary in writing if it becomes aware that the drinking water it is supplying to another person does not comply, or is not likely to comply, with any relevant water quality standard and must do so within 10 days after it becomes aware of that fact.

If any routine drinking water samples for a water sampling area did not, or were not likely to, comply with the relevant water quality standard, present and detail the results. For each water quality standard not met, detail the actions taken to manage the event; the corrective actions taken to return drinking water to specification and preventive measures implemented to reduce risk of future recurrence. Related reports made under both s. 18 and s. 22 should be acknowledged and a cross-reference made to the details provided under Emergency incident and event management section or vice versa to avoid duplication in the report.

More specific guidance on reporting of Regulation 16 standards is provided below.

## Presentation of Regulation 16 Drinking water quality standards

#### Regulation 16 (2) (a) Escherichia coli

Any E. coli detection must be reported under s. 22, s18 and r. 20.  Detail detections under the Emergency, incident and event management section in the annual report. Additionally, water suppliers should detail the actions taken in response to the contravention or risk.

All routine E. coli sample results should be recorded as shown in Table 4.

Table 4: Example table for Escherichia coli

| Water sampling area | Frequency of sampling | Number of samples | Maximum detected (orgs/100mL) | Number of known or suspected E. coli contamination events (s. 22/s18) * |
| --- | --- | --- | --- | --- |
| Area A | Weekly | 52 | 1 | 2 |
| Area B | Twice weekly | 104 | 1 | 3 |

Standard: All samples of drinking water collected are found to contain no Escherichia coli per 100 milliliters of drinking water

*Ensure these are discussed in the Emergency incident and event management section.

Total trihalomethanes

Safe Drinking Water Regulations 2025 Total trihalomethanes Less than or equal to 0.25 milligrams per litre of drinking water

All water sampling program sample results for total trihalomethanes should be presented as shown in Table 5. Any relevant s. 18 notifications require detail on how the standard was not met and on the corrective and preventive actions undertaken.

Table 5: Example table for total trihalomethanes

| Water sampling area | Frequency of sampling | Number of samples | Drinking water quality standard (mg/L) | Maximum (mg/L) | Average (mg/L) | Number of samples where standard was not met (s. 18) |
| --- | --- | --- | --- | --- | --- | --- |
| Area A | Monthly | 11* | 0.25 | 0.26 | 0.17 | 1# |
| Area B | Monthly | 12 | 0.25 | 0.15 | 0.08 | 0 |

Standard: Total trihalomethanes less than or equal to 0.25 milligrams per liter of drinking water.

* One missed sample due to laboratory scheduling error

# Detail this event under Drinking water quality standard not met and actions undertake’ underneath the THM results table.

## Aesthetic characteristics: r. 21 (n)

#### pH

Safe Drinking Water Regulations 2025 Regulation 21 – Additional details to be included in annual reports of water suppliers (n) a summary of steps taken by the water supplier to manage the aesthetic characteristics of the drinking water supplied by the water supplier during the financial year, including steps taken to manage the taste, odour, clarity and pH of that water.

Drinking water parameters monitored to manage aesthetics such as pH, alkalinity, hardness, iron and colour, should be reported as per Table 6. Further information, such as any aesthetic operational benchmarks set should be included, along with an explanation of how this aesthetic benchmark has been derived (e.g. using maximum value, mean value, percentile or confidence intervals, etc.).

Table 6: Example table for pH

| Water sampling area | Frequency of sampling | Number of samples | Minimum | Maximum | Aesthetic operating range |
| --- | --- | --- | --- | --- | --- |
| Area A | Monthly | 12 | 6.7 | 8.1 | 6.5 – 8.5 |

Note: the minimum result is optional for parameters other than pH, where both a minimum and maximum should be listed.

If there were issues in the reporting period with the taste, odour, colour or pH of the drinking water supplied, or there have been instances where the drinking water did not satisfy any aesthetic guideline values (either as set out in the Australian Drinking Water Guidelines or determined as appropriate by the water supplier), include a short discussion on how the issue was managed, including the cause, corrective actions taken in response and long term preventive measures.

#### Turbidity

Safe Drinking Water Regulations 2025 - Regulation 16(3)– Drinking water quality standards Turbidity For all samples collected from a water sampling area during a period of 12 months and tested for turbidity levels, the 95th percentile of results for that period does not exceed 5.0 Nephelometric Turbidity Units (NTU).

All water sampling program sample results for turbidity during a 12-month period should be presented as shown in Table 7. Relevant s. 18 notifications require detail on why the standard was not met along with corrective and preventive actions undertaken.

Note: comparison of the turbidity results against the previous two financial years should be based on the corresponding set of results and not on previous reported compliance performance.

Table 7: Example table for turbidity

| Water sampling area | Sampling frequency | Number of samples | Maximum turbidity in a sample (NTU) | Maximum 95th percentile of turbidity results in a 12 month period (NTU) | Number of 95th percentile of results above standard (s. 18) |
| --- | --- | --- | --- | --- | --- |
| Area A | Weekly | 52 | 16.4 | 5.1 | 1# |
| Area B | Weekly | 52 | 4.8 | 0.8 | 0 |

Standard: The 95th percentile of results for samples in any 12-month period must be less than or equal to 5.0 Nephelometric Turbidity Units (NTU).

# Discuss this event under Drinking water quality standard not met & actions undertaken underneath the turbidity results table.

## Presentation of other water quality standards (microbial, chemical or physical characteristic) - r. 16(2)(b) and (c)

Regulation 16 (2) (b) requires water suppliers to ensure drinking water supplied does not contain a chemical or physical characteristic specified in Table 10.6 of the Australian Drinking Water Guidelines, in that the amount of characteristic in the drinking water does not exceed the health guideline value (if any) in Table 10.6.

Regulation 16(2) (c) requires water suppliers to ensure drinking water supplied that contains any microbial, chemical, or physical characteristic not referred to in r. 16(2) (a) or (b), that the amount of the characteristic is less than an amount that would pose a risk to human health, whether that risk arises from the characteristic by itself or in combination with other characteristics.

These water quality standard results should be presented in the annual report as per Table 3.

Table 8 and 9 provide further examples of the suggested reporting format.

Table 8: Example table for copper

| Water sampling area | Sampling frequency | Number of samples | Drinking water quality standard (mg/L) | Maximum (mg/L) | Average (mg/L) | Number of samples where standard was not met (s. 18) |
| --- | --- | --- | --- | --- | --- | --- |
| Area A | Monthly | 12 | 2 | 0.2 | 0.1 | 0 |

Standard: The total concentration of copper in drinking water should not exceed 2.0 mg/L.

Table 9: Example table for fluoride

| Water sampling area | Sampling frequency | Number of samples | Drinking water quality standard (mg/L) | Target optimum operating fluoride concentration (mg/L) | Maximum (mg/L) | Average* (mg/L) | Number of samples where standard was not met (s. 18) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Area A | Monthly | 12 | 1.5 | 0.8 | 1.2 | 0.8 | 0 |

Standard: The total concentration of fluoride in drinking water should not exceed 1.5 mg/L.

* Note: under s. 5(3) of the Health (Fluoridation) Act 1973 fluoride added to drinking water must not result in an average optimum concentration in excess of one part fluoride per million parts of water.

Aesthetic drinking water parameters tested as part of the water supplier‘s water sampling program such as iron, hardness, pH and colour are to be reported under an Aesthetic characteristics section.

## Water quality complaints r. 21(m)

Safe Drinking Water Regulation 2025 Regulation 21 – Additional details to be included in annual reports of water suppliers (m) a summary of - (i) any complaints received by the water supplier during the financial year that related to the quality of drinking water supplied by the water supplier; and (ii) the water supplier’s responses to those complaints and analysis of any issues that arose from those complaints.

A complaint is defined as a written or verbal expression regarding an issue with the quality of the drinking water, which requires resolution.

The number of complaints received by type and water sampling area should be presented as indicated in Table 10 and 11. The type of complaints categories reported should be the types used by the water supplier.

Detail outlining any contributing factors, the general response to customer complaints, and any corrective actions undertaken should be included. Media releases and announcements made as a result of complaints should also be detailed in this section.

An analysis of the issues arising from the complaints and a comparison with previous reporting periods should be provided, stating whether there has been an increase, decrease or no change.

Table 10: Example table for types of complaints compared to previous reporting periods

| Type of complaints | Number of complaints |  |  | Comparison with previous reporting periods | Comments |
| --- | --- | --- | --- | --- | --- |
|  | Current reporting period | Previous A reporting period | Previous B reporting period |  |  |
| Alleged illness | 3 | 3 | 2 | No significant change | Nil. |
| Dirty water | 128 | 93 | 85 | Increase of thirty-five complaints from previous reporting period. | A valve malfunction caused reverse water flow through the reticulation system for the E water sampling area. Forty-three widespread public complaints were received. Refer to s. 22 report in the ‘Emergency, incident and event management’ section for further details. |
| Taste or odour | 34 | 6 | 10 | Increase of twenty-eight complaints from previous reporting period. | MIB and geosmin found in drinking water in the C water sampling area. Treatment Plant D does not have powdered activated carbon dosing available. |
| White water | 10 | 6 | 2 | Increase of four complaints from previous reporting period. | Six complaints due to a water main burst event in B water sampling area in January 2017. |
| Other | 8 | 10 | 9 | Types of other complaints not comparable to previous period. | Complaints included internal plumbing issues. Corrosion of copper galvanised pipes. |

Table11: Example table for types of complaints by water sampling area

| Water sampling area | Type of complaint |  |  |  |  | Total complaints |
| --- | --- | --- | --- | --- | --- | --- |
|  | Alleged illness | Dirty water | Taste or odour | White water | Other |  |
| Area A | 0 | 25 | 0 | 2 | 2 | 29 |
| Area B | 1 | 14 | 2 | 6 | 0 | 23 |
| Area C | 0 | 13 | 21 | 0 | 3 | 37 |
| Area D | 1 | 30 | 10 | 1 | 1 | 43 |
| Area E | 1 | 46 | 1 | 1 | 2 | 51 |

## Risk management plan audit results r. 21(e)

Safe Drinking Water Regulations 2025 Regulation 21 – Additional details to be included in annual reports of water suppliers - a summary of the findings of the most recent risk management plan audit of the water supplier’s risk management plan and any issues that the approved auditor raised during the risk management plan audit.

If a risk management plan audit was conducted during the reporting period, the outcome and findings of the audit must be stated, including improvement opportunities and observations raised by the auditor.

Include a copy of the audit certificate in the report.

A progress summary of actions taken by the water supplier during the reporting period should be included that addresses any identified improvement opportunities and observations made by auditors during previous risk management plan audits.

## Undertakings r. 21(d)

Safe Drinking Water Regulations 2025 Regulation 21 – Additional details to be included in annual reports of water suppliers - If the Secretary accepted a written undertaking by the water supplier under s.30 of the Act during the financial year, a summary of that undertaking.

Provide a summary of any undertakings which were in effect during the reporting period. Include detail on why an undertaking was sought, activities conducted (or planned) to alleviate the issue, timeframes, milestones and progress. Related community or stakeholder consultation activities should also be provided.

The summary may be presented as shown in Table 12. If there were no undertakings in place during the reporting period, this section can be excluded from the report.

Table 12: Example table for undertakings - Water supplier

| Water supply | Nature of contravention | Proposed actions to be taken to rectify contravention | Interim risk management actions | Start date | End date | Status at 30 June |
| --- | --- | --- | --- | --- | --- | --- |
| Area A | Multiple water quality standard failure for X parameter | [Details] | [Details] | 1/1/15 | 30/3/17 | Completed |

## Aesthetic standard variations r. 21(k)

Safe Drinking Water Regulations 2025 Regulation 21 – Additional details to be included in annual reports of water suppliers (k) If a variation of an aesthetic standard was approved under Section 19 of the Act in respect of drinking water supplied by the water supplier during a period of the financial year, a summary of - - that variation; and - any conditions imposed under Section 21 of the Act in relation to that variation.

This regulation refers to variations of aesthetic standards. If there were no s. 19 variations in place nor any s. 21 notices issued during the reporting period, this can be excluded from the report.

## Exemptions r. 21(l)

Safe Drinking Water Regulations 2025 Regulation 21 – Additional details to be included in annual reports of water suppliers (l) If an exemption from a water quality standard was approved under Section 20 of the Act in respect of drinking water supplied by the water supplier during a period of the financial year, a summary of - - that variation; and - any conditions imposed under Section 21 of the Act in relation to that variation.

This regulation refers to exemptions from water quality standards. If there were no s. 20 exemptions in place and no s. 21 notices issued, this can be excluded from the report.

## Regulated water r. 21(o)

Safe Drinking Water Regulations 2025 Regulation 21 – Additional details to be included in annual reports of water suppliers (o) If the water supplier supplied regulated water during the financial year, details of – - that regulated water. - the declaration under section 6 of the Act in respect of that regulated water; and - the steps taken in accordance with section 25 of the Act.

Details of regulated water supplies are to be included in the report. The definition of regulated water is water that is not intended for drinking but could reasonably be mistaken as drinking water. Details about the supplies must be provided as shown in Table13. Where a regulated water declaration supply relates to water that was previously a drinking water supply, this change needs to be explained.

If no regulated water was supplied during the reporting period, indicate this was the case.

Table 13: Regulated water supplies

| Regulated water supply | Source water | Gazettal date | Population supplied |
| --- | --- | --- | --- |
| Town X | Brown Creek | 1/1/15 | 800 |

Water suppliers are required under s. 25 of the Act to report on the steps taken to make the public aware of regulated water.

Safe Drinking Water Act 2003 Section 25 – Warnings to be given if regulated water supplied A water supplier who supplies regulated water must take all reasonable steps to ensure that the intended recipients of the water are made aware of the nature of the water and of the health risks that may arise from the use of the water.

Provide a summary that describes how intended users of regulated water are made aware of the quality of the water and the potential health risks from the use of that water. The summary may include reference to:

- Signage warning the public that the regulated water is not fit for drinking

- Management of publicly accessible taps so regulated water cannot be accidentally consumed

- Communication strategies and messages to advise all customers, particularly community stakeholders, customers operating food premises, accommodation premises, healthcare facilities and schools, that the water is not intended for drinking

- Management of supply where water is not fit for other non-drinking purposes, such as showering.

# Part 2: Water storage manager content guide

## Overview

Describe the overall water supply systems (from catchment-to-tap), the commitment to drinking water quality and the preventive measures applied to manage source water risks.

## Water supply systems

Provide a summary of each water supply system including where the raw water for each water supplier is sourced, from the catchment to the point of supply. A table similar to Table 14 can be provided to assist readers to better understand the water supply system serviced.

State whether water supplied to a water supplier has been treated before supply or supplied as raw water.

Include a clear map or diagram of the water supply systems serviced. Detailed process control schematics are not required.

Highlight any changes to source water supply arrangements during the reporting period from the previous period. Include any other information the water storage manager considers relevant, for example water quality improvement monitoring programs, water catchment projects and research projects.

Table 14: Example table for water supply systems

| Catchments |  |  | Treatment | Treatment plant | Treatment storages | Water suppliers |
| --- | --- | --- | --- | --- | --- | --- |
|  | Source water | Raw water storage area |  |  |  |  |
| Catchment Y | River Y | Storage Y | No | N/A | Treatment Water Storage Tank A | C Water Agency |
| Catchment X | River X | Storage X | Yes | E Treatment plant | Treatment Water Storage Tank B | D Water Agency E Water Agency |

## Source water

Water storage managers should describe the various source waters it manages, detail the catchment management authorities and water suppliers for its water supply systems. This may include in general terms the nature, purpose and comprehensiveness of the raw water sampling program (e.g. raw water storages, natural water courses, catchments or reservoirs) for pathogens, harmful algae, chemicals and radiological parameters, and interactions with other agencies regarding catchment management activities. For water storage managers where drinking water treatment processes were applied to raw water, provide details of methods used in sanitary surveys and to quantify microbial hazards.

## Drinking water treatment processes r. 22(e)

Safe Drinking Water Regulations 2025 Regulation 22 – Additional details to be included in annual reports of water storage managers - if the water storage manager applied a drinking water treatment process to water supplied by the water storage manager during the financial year – - a summary of how the water was disinfected or treated; and - a summary of any other processes applied to the water; and - a summary of any issues that arose as a consequence of applying those processes; and - a list of all chemicals and other substances used to disinfect or treat the water

Regulation 22(e) requires a summary of details of the drinking water treatment processes applied to the water supplied by a water storage manager in the annual report, any issues that arose as a result of those processes and must include a list of all chemicals used to disinfect and treat the water supplied.

If the water storage manager does not apply any drinking water treatment processes before supplying its water suppliers, indicate this is the case.

Information describing the treatment processes, changes made and any periodic treatment activities are to be included for each drinking water supply system. This description should extend from the point raw water enters the treatment plant through to the water supplier off-take point.

Information on the treatment processes and the chemicals associated with those processes (including water-soluble packaging that may be used to add chemicals) should be presented as shown in Table 15. Changes in the water treatment process between the previous and this reporting period should also be highlighted.

Table 15: Example table for drinking water treatment processes - Water storage manager

| Water supply system | Treatment plant |  |  | Role of each process |
| --- | --- | --- | --- | --- |
|  |  | Treatment process^ | Added substance/s |  |
| X supply system | D treatment Plant | Chlorination Fluoridation | Chlorine gas Fluorosilicic acid | Disinfection Provide oral health benefit |

^ See Appendix 1 for the level of requested detail in relation to water treatment processes.

## Summary of any issues

Regulation 22(e)(iii) requires any issues from the application of treatment processes to be detailed.

These issues may include situations where there was insufficient treatment due to treatment process breakdowns and occasions where there was loss of control of treatment processes leading to exceedances of drinking water quality standards of water suppliers.

Examples include clarifier malfunction leading to sub-optimal coagulation/flocculation, continued operation of treatment processes following breaches of critical control point critical limits, alternative drinking water supply sought by the water storage manager due to poor raw water quality or delay with treatment process repairs. This discussion is to include information on the effectiveness of any interim and long‑term interventions, timing of remedial works or activities and any impacts on the quality of drinking water supplied. Issues reported under s. 22 of the Act can be acknowledged in this section and cross referenced to details provided under the Emergency incident and event management section of the report. If there were no issues, indicate that this was the case.

## Emergency incident and event management r.22(a) & 22(b)

Safe Drinking Water Regulation 2025 Regulation 22 – Additional details to be included in annual reports of water storage managers - If an emergency or incident occurred during the financial year that affected the quality of water supplied by the water storage manager, information about actions taken by the water storage manager in response to that emergency or incident; and - any issues that may have arisen out of the consequence of the actions referred to in paragraph (a).

Regulation 22(a) and 22(b) requires information, actions and issues relating to emergencies, incidents and management of drinking water quality events by a water storage manager to be included in its annual report.

Details reported for any emergency, incident or event, which affected both the water supplier and water storage manager, should not contradict information contained in each water agency‘s annual report.

## Known or suspected contamination reported under s. 22

Information on known or suspected contamination reported under s. 22 must be included.

For each report made to the Department under s. 22, provide context of the event in relation to the affected water suppliers and water sampling areas. Clearly describe the event and include the issues identified, the corrective actions taken to alleviate the issue and to reduce risk, and any preventive actions initiated to minimise the risk of the issue recurring.

Safe Drinking Water Act 2003 Section 22 – Officer to report known or suspected contamination - This section applies if an officer of a water supplier, water storage manager or council believes or suspects, on reasonable grounds, that water supplied, or to be supplied, for drinking purposes— - may be the cause of an illness; or - may be the means by which an illness is being, has been or will be, transmitted; or - may contain any pathogen, substance, chemical or blue-green algae toxin, whether alone or in combination, at levels that may pose a risk to human health; or - may cause widespread public complaint. On forming that belief or suspicion, the officer must immediately report his or her belief or suspicion to the Secretary and must make the report in the form required by the Secretary.

## Issues from emergency or incident

Under r. 22(b) any issues that occurred from any actions referred to in r. 22(a) must be detailed.

## Presentation of information

Each situation should be clearly identified, described in narrative text and contained in the main body of the report. These situations are not to be reported solely in footnotes. A table summarising the information may be included.

The following information should be provided where applicable:

- The date, estimated duration and the nature of the emergency, incident, or event

- Affected water suppliers and drinking water supplies which were affected

- The actions taken in response to correct the issues or reduce risks

- Preventive measures implemented to minimise the risk of the issue recurring, including those where the water storage manager has implemented across its entire supply network

- The nature of communication with affected communities and/or water suppliers.

## Risk management plan audit results r. 22(d)

Safe Drinking Water Regulation 2025 Regulation 22 – Additional details to be included in annual reports of water storage managers (d) a summary of the findings of the most recent risk management plan audit of the water storage manager’s risk management plan and any issues that the approved auditor raised during the risk management plan audit.

If a risk management plan audit was conducted during the reporting period, the outcome and findings of the audit must be stated, including improvement opportunities and observations raised by the auditor. Include a copy of the audit certificate in the report.

A progress summary of actions taken by the water storage manager during the reporting period should be included which addresses any identified improvement opportunities and observations made by auditors during previous risk management plan audits.

## Undertakings r. 22(c)

Provide a summary of any undertakings which were in effect during the reporting period. Include detail on why an undertaking was sought, activities conducted (or planned) to alleviate the issue, timeframes, milestones and progress. Related community or stakeholder consultation activities must also be provided.

Safe Drinking Water Regulation 2025 Regulation 22 – Additional details to be included in annual reports of water storage managers (c) if the Secretary accepted a written undertaking by the water storage manager under section 30 of the Act during the financial year, a summary of that undertaking.

The summary may be presented as shown in Table 16. If there were no undertakings in place during the reporting period, this section can be excluded from the report.

Table 16: Example table for undertakings - Water storage manager

| Water supply | Describe contravention | Proposed actions to be taken | Interim risk management actions |  |  | Status at 30 June |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Start date | End date |  |
| System A | Communication failures | [Details] | [Details] |  |  | In progress |
|  |  |  |  | 1/1/24 | 30/12/25 |  |

## Exemptions r. 22(f)

Safe Drinking Water Regulation 2025 Regulation 22 – Additional details to be included in annual reports of water storage managers f) if a water storage manager was exempted under section 8(3) of the Act from compliance with section 8(1) of the Act during a period of the financial year, details of that exemption and any condition imposed in relation to that exemption.

This regulation refers to an exemption from preparing, implementation, and review of a risk management plan. If there were no s. 8 exemptions in place during the reporting period, this can be excluded from the report.

# Appendix 1: Example template for reporting drinking water treatment processes

|  |  | Drinking water treatment processes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Clarification |  |  | Filtration |  | Disinfection |  |  |  |  | Other |  |  |  | Added substances |  |  |  |  |  |  |
|  |  | Coagulation and flocculation | Sedimentation or clarification | Dissolved air flotation | Granular media filter | Membrane | Chlorine gas | Sodium hypochlorite | Chlorine dioxide | Ultraviolet (UV) | Ozone | Activated carbon (PAC/GAC) | Ion exchange | Reverse osmosis | Sludge-handling (mechanical with chemical addition) | Lime/soda ash/caustic soda/ carbon dioxide/sulphuric acid | Aluminium-based coagulants | Iron-based coagulants | Polymers | Chlorine | Ammonia | Fluoride |
| Water treatment plant | A B |  | x |  |  |  | x |  |  |  | x | x |  |  | x | x | x |  | x | x |  | x |
|  |  |  |  |  |  | x | x |  |  |  | x | x |  |  | x | x | x |  | x | x |  | x |
|  | C D | x |  | x | x |  | x |  |  | x |  | x |  |  | x | x | x |  | x | x | x | x |
|  |  | x | x |  |  |  | x |  |  | x |  | x | x |  | x | x | x |  | x | x | x | x |
|  | E F | x | x | x | x |  |  | x |  |  | x |  | x |  |  | x | x |  | x | x |  |  |
|  |  | x | x |  |  |  |  |  | x |  |  |  |  | x |  | x |  | x |  |  |  | x |

# Appendix 2: Checklist for water suppliers

### Overview

- State commitments to drinking water quality and proactive management of water quality risks.

- Summarise water supply systems including a map or diagram and Table 1 - Water supply systems.

- Discuss any major changes to water supply arrangements since the previous reporting period.

- Discuss source waters in supply systems and (if applicable) discuss how source water risks are quantified and managed.

### New or variations to water sampling areas r. 21 (a)

- Include details of any new water sampling areas and any variations to water sampling areas specified in the water supplier’s risk management plan

### Drinking water treatment processes r. 21(f)

- Include Table 2 - Drinking water treatment processes - Water supplier and Appendix 1 Table

- Describe the role of each treatment process and chemical additive (or include in glossary of terms)

- Discuss any major changes to treatment processes since previous reporting period.

- Discuss any issues arising from treatment processes.

- Acknowledge reports made under s. 22 and cross-reference to Emergency incident and event management

### Emergency incident and event management r. 21(b) & 21(c)

- Include information on each situation reported under s. 22.

- Include relevant information on the actions taken, issues experienced and preventive measures identified with each situation reported.

### Presentation of information

### Drinking water quality standards Regulation 21(g), (h), (i) & (j)

- in relation to the water supplied by the water supplier during the financial year, details of the water’s compliance with drinking water quality standards (as specified in regulation 16 for the purposes of section 17(1) of the Act); and

- details of the water supplier’s compliance with regulation 17 during the financial year; and

- if water supplied by the water supplier during the financial year, did not comply with a drinking water quality standard, information about actions taken by the water supplier in relation to that water; and

- in relation to the analysis of water samples collected by the water supplier during the financial year:

  - an evaluation of analysis results and information in respect of those water samples; and

  - a comparison of those results and information with corresponding results and information in respect of water samples collected during the previous 2 years

### Escherichia coli

- Include Table 4 - E. coli standard and a compliance analysis of the E. coli results against the standard for the reporting period.

- Provide a comparison of the E. coli results from the reporting period with the two previous financial years.

- Discuss any variations to the sampling frequency specified in the drinking water sampling program or gazetted in accordance with r. 17.

- If any samples were not taken at the frequency specified in the water sampling program and/or analysed, provide further explanations.

## Trihalomethanes

- Include Table 5 - Trihalomethanes standard and a compliance analysis of the total trihalomethanes results against the standard for the reporting period.

- Provide a comparison of the trihalomethanes results from the reporting period with the two previous financial years.

- Discuss any variations to the sampling frequency specified in the drinking water sampling program or gazetted in accordance with r. 18.

- If any samples were not taken at the frequency specified in the water sampling program and/or analysed, provide further explanations.

## pH

- Include Table 6 – Detail pH results against the standard for the reporting period.

- Provide a comparison of the pH results from the reporting period with the two previous financial years.

## Detail variations to the sampling frequency specified in the drinking water sampling program

- Discuss any variations to the sampling frequency specified in the drinking water sampling program or gazetted in accordance with r. 18.

- If any samples were not taken at the frequency specified in the water sampling program and/or analysed, provide further explanation.

## Turbidity

- Include Table 7 - Turbidity standard and a compliance analysis of the turbidity results against the standard for the reporting period.

- Provide a comparison of the turbidity results from the reporting period with the two previous financial years.

- Discuss any variations to the sampling frequency specified in the drinking water sampling program or gazetted in accordance with r. 18.

- If any samples were not taken at the frequency specified in the water sampling program and/or analysed, provide further explanations.

## Other water quality standards r. 16 (microbial, chemical, or physical characteristics)

- Include Table 3 for each r. 16 drinking water quality standard monitored from the drinking water sampling program and a compliance analysis for each set of drinking water quality standard results against the relevant standard for the reporting period.

- Provide comparisons of each drinking water quality standard results from the reporting period against the corresponding sets of results for the two previous financial years.

- Discuss any variations to each relevant standard sampling frequency specified in the drinking water sampling program or gazetted in accordance with r. 18.

- If any samples were not taken at the frequency specified in the water sampling program and/or analysed, provide further explanations.

## Drinking water quality standards not met & actions undertaken

- Where a relevant standard was not met and reported under s. 18, include details of the results, actions and preventive measures taken underneath each standard table. Cross reference to s. 22 in Emergency, incident and event management section (if applicable)

## Aesthetic characteristics r. 16(1)(b)

- Include Table 10 - Aesthetic parameters for each water sampling area.

- If applicable, discuss any issues regarding taste, odour, clarity or pH during reporting period.

- Include a brief summary of steps taken to manage aesthetic issues.

## Water quality complaints r. 21(m)

- Define complaint and include Table 11 and 12

- Detail general response, contributing factors, corrective actions, community meetings, public communications and media releases issued in response to customer complaints.

- Provide an analysis of issues arising from the complaints received and a comparison of previous financial years.

## Aesthetic characteristics r. 21(n)

- Include a summary of steps taken by the water supplier to manage the aesthetic characteristics of the drinking water supplied by it, including steps taken to manage the taste, odour, clarity and pH of the drinking water supplied.

## Risk management plan audit results r. 21(e)

- If a risk management plan audit was conducted during the reporting period, discuss whether the risk management plan met the obligations of the Act during the audit period and include the audit certificate in the report.

- Summarise the findings of the audit.

- Describe any specific improvement opportunities and observations identified by the auditor.

- Report on progress with identified improvement opportunities from previous audits (if applicable)

## Undertakings r. 21(d)

- Include Table 13 - Undertakings and provide a summary of relevant information related to undertakings (if applicable)

## Aesthetic standard variations r. 16(1)(c) r. 21(k)

- Provide details of any variations to aesthetic standards granted under s. 19 of the Act (if applicable)

## Exemptions r. 21(l)

- Provide details of any exemptions granted under s. 20 of the Act (if applicable)

## Regulated water r. 21(o)

- Include definition of regulated water‘ provided in guidance.

- Include Table 14 - Regulated water supplies (if applicable)

- Discuss any regulated water supplies which were previously regarded as drinking water (if applicable)

- Summarise regulated water awareness and risk management activities undertaken under s. 25 (if applicable)

# Appendix 3: Checklist for water storage managers

## Overview

- State commitments to drinking water quality and proactive management of source water risks.

- Summarise water supply systems including a map or diagram and Table 15 - Water supply systems.

- Discuss any major changes to water supply arrangements since the previous reporting period.

- Discuss source waters in supply systems and (if applicable) discuss how source water risks are quantified and managed.

## Drinking water treatment processes r. 22(e)

- State if water supplied to a water supplier has been treated before supply or is supplied as raw water.

- Include Table 16 - Drinking water treatment processes - Water storage manager and Appendix 1 Table

- Describe the role of each treatment process and chemical additive (or include in glossary of terms)

- Discuss any major changes to treatment processes since previous reporting period.

- Discuss any issues arising from treatment processes.

- Acknowledge reports made under s. 22 and cross-reference to ‗Emergency incident & event management ‘

## Emergency incident and event management r. 22(a) & 22(b)

- Include information on each situation reported under s. 22.

- Include relevant information on the actions taken, issues experienced and preventive measures identified with each situation reported.

- Discuss other events not reportable under s. 22 of the Act under separate headings.

## Risk management plan audit results r. 22(d)

- If a risk management plan audit was conducted during the reporting period, discuss whether the risk management plan met the obligations of the Act during the audit period and include the audit certificate in the report.

- Summarise the findings of the audit.

- Describe any improvement opportunities and observations identified by the auditor.

- Report on progress with identified improvement opportunities from previous audits (if applicable)

## Undertakings r. 22(c)

- Include Table 17 - Undertakings and provide a summary of relevant information related to undertakings.

## Exemptions r. 22(f)

- Provide details of any exemptions granted under s. 8 of the Act (if applicable).

Acknowledge any matters relevant to the 2015 Regulations for those five days, if any, in a footnote.